| Technical Specifications / Teknik Özellikler |  |
| --- | --- |
| Operating Temperature / Çalışma Sıcaklığı | 90°C |  |
| Konstrüksiyon / Yapım | MTSEF: Aluminum Cap/Galvanized coated sheet metal cover+Stud reinforcement Alüminyum başlık/Galvaniz kaplı saç kapak+Saplama takviyesi MTNEF: Aluminum cap/Galvanized coated sheet metal cover. Alüminyum başlık/Galvaniz kaplı saç kapak. |  |
| Adhesive Material / Yapıştırıcı malzeme | Polyester Adhesive Katkı maddeli döküm polyester |  |
| Flange / Bağlantı | G1"-G1 1/2"-G2"- G2 1/2"-G3" |  |
| Flow Direction / Akış Yönü | Outside to Inside Dıştan İçe |  |
| Filter Material / Filtre Malzemesi | Chrome nickel wire Krom nikel tel |  |
| Nominal Flow Rate / Nominal Akış Oranı | 15~1400Lt/min 15~1400lt/dk. |  |
| Filter Sensitivity / Filtre Hassasiyeti | 125 µ |  |
| Internal support (mantene)/ İç destek (manten) | Phosphate Coated Sheet Fosfat Kaplı Sac |  |
| Compatibility with Fluids /Akışkanlar ile uyumluluğu | Note: The stated flow rates are 30cst. Viscosity is relative to oil. If the oil used/to be used in the environment is 30cst. If it is different from , determine the new flow from the table. Not: Belirtilen debiler 30cst. Viskozite yağa göredir. Eğer ortamda kullanılan/ kullanılacak yağ 30cst. Den farklı ise tablodan yeni debi tayin ediniz. HH-HL-HM- HR-HV-HG |  |
| Standards/Standartlar; | ISO 2941, ISO 2942, ISO 16889 |  |

| Dimensions / Boyutlar |  |  |  |  |
| --- | --- | --- | --- | --- |
| Filter Element Code / Filtre Elemanı Kodu | Flow LT / Debi LT |  | G | A |  | B | C | CH |
| MTSEF-015 | 15 |  | 1" | 88 |  | 66 | 59 | 42 |
| MTSEF-025 | 25 |  | 1" | 124 |  | 102 | 59 | 42 |
| MTSEF-045 | 45 |  | 1 1/2” | 164 |  | 142 | 59 | 42 |
| MTSEF-065 | 65 |  | 1 1/2” | 134 |  | 114 | 82 | 62 |
| MTSEF-090 | 90 |  | 1 1/2” | 188 |  | 168 | 82 | 62 |
| MTSEF-125 | 125 |  | 1 1/2” | 222 |  | 202 | 82 | 62 |
| MTSEF-150 | 150 |  | 1 1/2” | 215 |  | 196 | 97 | 62 |
| MTSEF-180 | 180 |  | 1 1/2” | 263 |  | 242 | 82 | 62 |
| MTSEF-225 | 225 |  | 1 1/2” | 192 |  | 167 | 130 | 80 |
| MTSEF-300 | 300 |  | 2” | 255 |  | 202 | 130 | 80 |
| MTSEF-500 | 500 |  | 2” | 282 |  | 254 | 130 | 86 |
| MTSEF-750 | 750 |  | 2 1/2” | 271 |  | 253 | 182 | 98 |
| MTSEF-1400 | 1400 |  | 3” | 523 |  | 504 | 182 | 98 |

*All Dimensions are in mm. / Tüm ölçüler mm olarak verilmiştir.

| Dimensions / Boyutlar |  |  |
| --- | --- | --- |
| Filter Element Code / Filtre Elemanı Kodu | Flow LT / Debi LT | G |  | A | B | C | CH |  |
| MTNEF-005 | 5 | 1/4" |  | 82 | 73 | 30 |  | 17 |
| MTNEF-015-12 | 15 | 1/2" |  | 84 | 72 | 64 |  | 43 |
| MTNEF-015 | 15 | 1" |  | 84 | 72 | 64 |  | 43 |
| MTNEF-025 | 25 | 1" |  | 119 | 107 | 64 |  | 43 |
| MTNEF-045 | 45 | 1" |  | 160 | 147 | 64 |  | 43 |
| MTNEF-065 | 65 | 1 1/2" |  | 132 | 116 | 90 |  | 65 |
| MTNEF-090 | 90 | 1 1/2" |  | 189 | 172 | 90 |  | 65 |
| MTNEF-125 | 125 | 1 1/2" |  | 222 | 206 | 90 |  | 65 |
| MTNEF-180 | 125 | 2" |  | 263 | 242 | 90 |  | 65 |

*All Dimensions are in mm. / Tüm ölçüler mm olarak verilmiştir.